DECOMPOSING COMPLEX, MACROSCOPIC PHENOMENA THROUGH A SET OF LOCAL NONLINEAR RULES

IN A CELLULAR AUTOMATA ENVIRONMENT

M.V. Avolio°, G.M. Crisci*, D. D’Ambrosio°, S. Di Gregorio°, R. Rongo*, W. Spataro°

°Department of Mathematics, *Department of Earth Sciences, Center of High-Performance Computing,

University of Calabria, Arcavacata, I-87036 Rende (CS)

# G. Iovine

## C.N.R.-I.R.P.I. via Cavour snc, 87030 Rende (CS), Italy

Contents:

Cellular Automata (CA) for modelling a-centric systems

### An empirical method for CA modelling complex macroscopic phenomena

### Two applications: SCIARA (lava flows) and SCIDDICA (debris flows)

### Results of simulations: Etna 2001 eruption and 1998 Sarno mudflows

Comments and conclusions.

C E L L U L A R   A U T O M A T A      ( C A )

| Intuitively a homogeneous Cellular Automaton (CA) can be seen as a d-dimensional space, partitioned in cells of uniform size, each one embedding an identical finite automaton, the elementary automaton (ea). |  |
| --- | --- |

Input for each cell is given by the states of the neighbouring cells, where the neighbourhood conditions are determined by a pattern invariant in time and space.

| 0,0 -1, 0 0, -1 1, 0 0, 1 -1, 1 -1, -1 1, -1 1, 1 -2, 1 -2, -1 -2, -2 -1, -2 0, -2 -2, 0 2, -2 1, -2 2, -1 2, 0 2,1 0, 0 1, 2 2,2 0, 2 -1, 2 -2, 2 von Neumann neighbouring | 0,0 0,1 -1,1 -1,0 0,-1 1,0 1,-1 0,2 0,-2 1,-2 2,-2 2,-1 2,0 1,1 -1,2 -2,2 -2,1 -2,0 -1,-1 hexagonal neighbouring |
| --- | --- |

At the time t=0, cells are in arbitrary states and the CA evolves changing the state at discrete times, according to the transition function.

AN  EMPIRICAL  METHOD  FOR  SIMULATING  MACROSCOPIC,  COMPLEX,  A-CENTRIC  PHENOMENA  BY  CELLULAR AUTOMATA

•	The cell corresponds usually to a portion of the space; so the cellular space must be three dimensional; two dimensions are sufficient, when the third dimension may be inserted in the state of the cell (e.g. the altitude in a phenomenon concerning the earth surface).

•	Two global parameters must be considered: the size of the cell and the clock of the CA transition; they effect usually the transition function implicitly.

•	Each characteristic, relevant to the evolution of the system and relative to the space portion corresponding to the cell, is individuated as a substate; the set Q of the states is given by the Cartesian product of the sets of substates:        Q = Q1  Q2  .......  Qn

•	As the state of the cell can be decomposed in substates, the transition function may be split in many “elementary” processes: internal transformations and local interactions:.

EMPIRICAL METHOD ASSUMPTIONS

First assumption: 	the substates are constant in the space occupied by the cell (e.g. the temperature).

The change of a substate can be caused only by                                                        internal transformations

interactions with the neighbourhood

Internal transformations are defined as the changes in the values of the substates due to cell internal conditions (substates inside the cell)

Local interactions are changes due to interactions of substates in the neighbourhood.

Second assumption: 	flows of some quantity, present in the cell and expressed as substate, must be described in terms of local interactions and must minimize locally unbalance conditions.

An algorithm for the minimisation of the differences and a relaxation rate must be determined opportunely.

Third assumption: 	the fragmentation of the complex phenomenon in internal transformations and local interactions may be applied, calculating sequentially the change of the values of the substates.

OUTFLOWS DETERMINATION BY THE MINIMISATION ALGORITHM

Local conditions for minimum unbalance conditions are determinant for the evolution of the system

Problem:		Outflows from the central cell to the other n neighbouring cells must be determined in

order to minimise the differences of a quantity q in the neighbouring cells:

Definitions		qd = quantity, that may be distributed, in the central cell

q0 = irremovable quantity in the central cell

qi = quantity in the cell i   1≤ i ≤ n

fi = outflows from the central cell    0in (f0 is the part of qd remaining in the central cell)

qi’ = qi + fi        0in

q’min is the minimum value for qi’  0in

Bounds		qd = i fi        0in

i (qi’- q’min)      must be minimised by the values of    fi   ( 0in )

Minimisation 	(a) All the neighbouring cells are “not eliminated”:  A is the set of not eliminated cells

algorithm 	(b) The “average q” (av_q) is found for the set A of not eliminated cells:

av_q = (qd + i qi)/#A    iA

(c) The cell x with qx > av_q is eliminated

(d) Go to step (b) until no cell is eliminated.

(e) fi = av_q -  qi    iA                 fi = 0    iA

Relaxation		The relaxation rate accounts for the determination of the part of flow that is effectively

rate r		transferred in the neighbour cell from the central cell in a CA step.

OUTFLOWS DETERMINATION BY THE MINIMISATION ALGORITHM

Minimisation 	(a) All the neighbouring cells are “not eliminated”:  A is the set of not eliminated cells

algorithm 	(b) The “average q” (av_q) is found for the set A of not eliminated cells:

av_q = (qd + i qi)/#A    iA

(c) The cell x with qx > av_q is eliminated

(d) Go to step (c) until no cell is eliminated.

(e) fi = av_q -  qi    iA                 fi = 0        iA

|  | 1 |  |
| --- | --- | --- |
| 3 | 0 | 2 |
|  | 4 |  |

EXAMPLE OF DISTRIBUTION (von Neumann neighbouring):

qd = 9,   q0 =  81,   q1 =  100,   q2 =  76,   q3 = 83,   q4 =  71,

|  | 100 |  |  |  | * |  |  |  | * |  |  |  | 100 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 83 | 81:9 | 76 |  | 83 | 81:9 | 76 |  | * | *9 | 76 |  | 83 | 81 | 76:2 |
|  | 71 |  |  |  | 71 |  |  |  | 71 |  |  |  | 71:7 |  |
| av_h=420/5=84 cell 1 is eliminated |  |  |  | av_h=320/4=80 cells 0 and 3 are eliminated |  |  |  | av_h=156/2=78 no cell is eliminated |  |  |  | f2=2 f4=7 f0=f1=f3=0 |  |  |

SCIARA: Simulation by Cellular Interactive Automata of the

Rheology of Aetnean lava flows (release hex1)

SCIARA = <R, X, L, Q, P, >

• R = {(x, y)| x, y N, 0  x  lx, 0  y  ly}  is the set of points with integer co-ordinates in the finite region, where the phenomenon evolves. N is the set of natural numbers.

• LR specifies the lava source cells

0,0

0,1

-1,1

-1,0

0,-1

1,0

1,-1

• X = {(0,0), (0,1), (0,1), (1,0), (-1,0), (-1,1), (1,-1)} is the set, which identifies the geometrical pattern of the cells, which influence the cell state change.

• The finite set Q of states of the ea:        Q = Qa  Qth  QT  Qo6  Qi6

| Qa | altitude of the cell |
| --- | --- |
| Qth | lava thickness in the cell |
| QT | lava temperature in the cell |
| Qo (Qi) | lava outflow (inflow) |

SCIARA (release hex1)

• P is the set of global parameters of SCIARA        P = {pc, pt, padh_v, padh_s, pTv, pTs, pr, pc}

| pc | side of the cell | 5 m |
| --- | --- | --- |
| pt | temporal correspondence of a step of SCIARA | 60 s |
| padh_v | lava adhesion at the vents | 0.7 m |
| padh_s | lava adhesion at the solidification | 10 m |
| pTv | lava temperature at the vents | 1373 K |
| pTs | lava temperature at solidification | 1123 K |
| pr | relaxation rate | 1 |
| pc | cooling parameter | 1.4 10-14 (m/K)3 |

• : Q7Q is the deterministic transition function of the CA

| elementary process in order | input | variations/determinations | updating |
| --- | --- | --- | --- |
| solidification | Qth  Qa  QT | (Qa , Qth) | Qa , Qth |
| lava outflows | (Qa  Qth)7  QT | Qo6 | Qo6 , Qi6 |
| lava mixing | (Qi  Qo)6 Qth  QT7 | Qth, QT | Qth, QT |
| lava cooling | Qth  QT | QT | QT |

• :LN Qth   specifies the emitted lava from the source cell at the step s.(sN)

SCIARA (release hex1) transition function 

Internal transformation 	SOLIDIFICATION		S: Qth  Qa  QT  Qa  Qth

0

2

1

4

5

3

6

The cell altitude remains unchanged until solidification condition holds: (QT < pTs),

then the altitude is increased by the lava thickness and lava thickness is zeroed.

Local interaction:		 LAVA OUTFLOWS		LO: (Qth  Qa)7  QT  Qo6

Adhesion computation:     adhesion = f(padh_v, padh_s, pTv, pTs, Qth, QT)

Minimisation	qd = quantity, that may be distributed, in the central cell = Qth - adhesion

algorithm		q0 = irremovable quantity in the central cell = Qa + adhesion

application	qi = quantity in the cell i   1≤ i ≤ 6 = Qa + Qth

Local interaction:	 	LAVA MIXING			LM: Qth Qi6  Qo6  Qr7  Qr

Lava mixing involves the determination for the central cell:

a) the remaining lava thickness (rem_th):		rem_th = Qth[0]-j Qo[j]       1j6

b) new lava thickness (new_th):			new_th = rem_th+j Qi[j]    1j6

c) the temperature variation by mixing is calculated as the average weight of QT, by considering both the remaining lava and the inflows: new_T = (rem_th* QT [0])+j(Qi[j]*QT[j]))/(rem_th+j Qi[j])   1j6

Internal transformation 	LAVA COOLING		LC: Qth  QT  QT

Temperature drop due to irradiation at the surface is computed, assuming that other losses are not relevant: newT=QT/31+(QT3⋅pcQth)

SCIARA (release hex1): simulation of the July 2001 Etnean eruption

The Etnean eruption started the 18th of July 2001

at an elevation of ca.2100 m, near Nicolosi (Sicily).

The lava threatened Nicolosi, however stopping after

10 days (maximum lava field length).

Simulation with SCIARA were carried to develop

future scenarios and/or possible human intervention.

Simulation of the eruption first four days                       The first 10 days of the event: comparison

between simulated (right) and real event (left).

SCIDDICA: Simulation through Computational Innovative methods for the Detection of Debris flow path using Interactive Cellular Automata  (release S3-hex)

run-up

altitude

landslide debris

kinetic head

detrital cover

carbonate rocks

SCIDDICA = <R, X, Q, P, >

0,0

0,1

-1,1

-1,0

0,-1

1,0

1,-1

• R = {(x, y)| x, y N, 0  x  lx, 0  y  ly}  is the set of points with integer co-ordinates in the finite region, where the phenomenon evolves. N is the set of natural numbers.

• X = {(0,0), (0,1), (0,1), (1,0), (-1,0), (-1,1), (1,-1)} is the set, which identifies the geometrical pattern of the cells, which influence the cell state change.

• The finite set Q of states of the ea:        Q = Qa  Qth  Qr  Qd  Qo6  Qi6

| Qa | altitude of the cell (related to bedrock and depth of detrital cover) |
| --- | --- |
| Qth | thickness of landslide debris |
| Qr | run-up height (it is a function of the kinetic energy of the landslide and expresses the height that can be overcome by the flowing debris) |
| Qd | depth of detrital cover that can be eroded and hence transformed into landslide debris |
| Qo (Qi) | debris outflow (inflow) |

SCIDDICA (release S3-hex)

• P is the set of global parameters of SCIDDICA        P = {pc, pt, padh, pt, pr, prl, pmt, ppef}

| pc | apothem of the hexagonal cell | 1.5 m |
| --- | --- | --- |
| pt | temporal correspondence of a step of SCIDDICA | 0.25 s (?) |
| padh | adhesion | 0.001 m |
| pf | elevation threshold for movement (related to friction angle) | 0.1 m |
| pr | relaxation rate of debris landslide outflows | 1 |
| prl | run-up loss | 0.6 m |
| pmt | activation threshold for mobilisation | 2 m2 |
| ppef | factor of progressive erosion | 0.04 |

• : Q7Q is the deterministic transition function of the CA

| elementary process in order | input | variations/determinations | updating |
| --- | --- | --- | --- |
| soil mobilisation | Qth  Qd  Qr | (Qa , Qth , Qd , Qr) | Qa , Qth , Qd , Qr |
| outflow of debris | (Qa  Qth)7  Qr | Qo6 | Qo6, Qi6 |
| debris mixing | Qth  Qi6  Qo6  Qr7 | Qth, Qr | Qth, Qr |
| energy loss | Qth  Qr | Qr | Qr |

SCIDDICA (release S3-hex) transition function 

Internal transformation 	MOBILISATION	M: Qth  Qd  Qr  Qa  Qth  Qd  Qr

The erosion of the detrital mantle is allowed, in proportion to the energy of the moving mass in the cell.

That is expressed by   Qth * Qr > pmt    .

The depth of eroded material, at each step, depends on the parameter ppef . It is the minimum value

between  1) the depth of available detrital cover Qd  and  2) the product ppefQthQr

Local interaction:	 DEBRIS OUTFLOWS		DO: (Qth  Qa)7  Qr  Qo6

Minimisation	qd = quantity, that may be distributed, in the central cell = Qr - padh

algorithm		q0 = inamovable quantity in the central cell = Qa + padh

application	qi = quantity in the cell i   1≤ i ≤ 6 = Qa + Qth

Local interaction:	DEBRIS MIXING		DM: Qth  Qi6  Qo6  Qr7  Qr

Debris mixing involves the determination for the central cell:

a) the remaining debris thickness (rem_th):		rem_th = Qth[0]-j Qo[j]       1j6

b) new debris thickness (new_th):			new_th = rem_th+j Qi[j]    1j6

c) the run-up variation by mixing is calculated as the average weight of Qr, by considering the remaining debris and the inflows:            new_r = ((rem_th* Qr [0])+j(Qi[j]*Qr[j]))/(rem_th+j Qr[j])   1j6

Internal transformation 	ENERGY LOSS 		EL: Qr  Qth  Qr

Energy loss , at each step, is computed in terms of run up loss and depends on the parameter prl. It is the minimum value between  1) the difference Qr - prl and  2) debris thickness Qth

SCIDDICA(release S3-hex) application to the Sarno landslides (the Curti case)

| On 5-6 May 1998, numerous rapid-extremely rapid debris flows were triggered by exceptional rainfalls in Campania, mostly on the slopes of Pizzo d’Alvano. Hundreds of small debris slides originated in the volcaniclastic mantle, and transformed into fast-moving debris flows. These latter generally eroded the entire depth of volcanic detrital cover, greatly increasing their initial volume. Landslides hit the urbanised areas, at the base of the slopes, killing 161 people and leaving more than 1,000 others homeless. A map of the disaster has been realised, at 1:5,000 scale: the geologic-geomorphologic context has been critically analysed, and the required information for simulation purposes acquired (e.g. pre-event conditions of slopes and detrital cover). Simulation data: Real area (R) = m2 96902 Simulation area (S) = m2 91093 RS=m270927 - RS=m2117017 (RS)/R=0.73 - (RS)/S=0.78 ((RS)/(RS))=0.78 |  |
| --- | --- |

COMMENTS  AND  CONCLUSIONS

- Cellular Automata represent an alternative approach to differential equations in modelling complex systems, whose evolution is strongly dependent on local interactions of their constituent parts.

- The empirical method, here introduced, was successfully applied by the research group “Empedocles” to other macroscopic complex phenomena, such as soil contamination and bioremediation, forest fires, soil erosion by rain; new application fields are considered: pyroclastic flows, marine environment evolution.

- This empirical method permits to start with simple models, whose refinement can be performed in an incremental way, introducing other internal transformations and local interactions. This allows a careful monitoring of the model building phase by comparison between real phenomena and simulations.

- This empirical method involves, for each internal transformation or local interaction, the introduction of problem-specific parameters, whose determination may be performed by applying optimization methods to minimize the difference between model results and experimental data. Genetic algorithms were effective for applications with several parameters.

- It is important to define the limits to the application of the model to similar phenomena: e.g., SCIARA was validated for the Etnean lava flows in 1986/7 eruption and 1991/2 eruption. SCIARA application during the eruption in the 2001 Summer for the hazard analysis was possible, because Etnean lavas features don’t change significantly in the time. Cases, where the features change, involve a validation considering an interval of possible values of parameters, corresponding to different typologies of cases.

- This point is crucial; investigantions showed that there are different confidence intervals for phenomena of the same type.

- A last consideration can be added: the decomposition of the complex macroscopic phenomenon in internal transformations and local interactions seems to have encouraged interdisciplinary cooperations and exchange of information, at least in the cases treated here.